IT 115 Networking 1

Laboratory Activity

Name: Date:

_           Course, Year & Section:

_           Remarks:

SIMULATING LAN CONNECTIONS USING STRAIGHT-THROUGH AND CROSSOVER CABLES IN CISCO PACKET TRACER

Objectives:

After completing this activity, you should be able to:

- Identify the correct type of cable for connecting networking devices.

- Create and configure a basic LAN in Cisco Packet Tracer.

- Assign IP addresses and verify network connectivity.

- Explain the difference between using straight-through and crossover cables.

Materials and Tools:

- Cisco Packet Tracer (you can download from this link: https://bit.ly/ciscopackettracer-dl )

- Computer with Packet Tracer installed

Activity:

Task 1. Using Straight-Through Cable

Steps:

- Open Cisco Packet Tracer.

- From the device list, drag and drop: 2 PCs (PC0, PC1) and 1 Switch

- Connect the devices:

- Use Copper Straight-Through cable between PC0 → Switch0

- Use Copper Straight-Through cable between PC1 → Switch0

- Assign the following IP addresses:

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |

- Go to PC0 → Desktop → IP Configuration to assign IP.

- Do the same for PC1.

- Test connectivity:

- Go to PC0 → Command Prompt, type: ping 192.168.1.2

- Record if ping replies are successful.

Task 2. Using Crossover Cable

Steps:

- Delete the switch and connect PC0 directly to PC1.

- Use Copper Cross-Over cable between the two PCs.

- Use the same IP addressing scheme as before.

- Test connectivity again using ping.

- Record your observation.

Task 3. Interchanging Cables

Steps:

- Recreate Scenario A (PC0 → Switch0 → PC1) but use a Copper Cross-Over cable instead of straight-through.

- Test connectivity using the ping command.

- Recreate Scenario B (PC0 → PC1) but use a Copper Straight-Through cable instead of crossover.

- Test connectivity using the ping command.

- Record your observations: which setups worked and which did not.

Task 4. Post activity questions

- Why did the straight-through cable work in the switch setup but not in a PC-to-PC setup?

- What happens if you use a straight-through cable between two PCs?

- What happens if you use the wrong cable type between two similar devices?

- What command is used to verify network connectivity in Packet Tracer?

Additional instructions:

- Use this activity material as your answer sheet.

- Include:

- Screenshots of your network topologies for Tasks 1, 2, and 3.

- Results of your ping tests and answers to post activity questions.

- Save your completed activity as PDF file and send it to noriannelamadrid@cnsc.edu.ph on or before November 9, 2025, 11:59PM.